This manuscript presents a novel and relevant contribution to process safety by introducing the concept of the jurisdiction compliance gap and proposing a Regulatory RAG–based verification layer to bridge engineering calculations and regulatory obligations. The topic is highly aligned with modern challenges in digitalisation, PSM compliance, and multi-standard environments (e.g., ASME/API vs national regulations such as KOSHA). The integration of cross-discipline validation and regulatory retrieval is particularly promising and could have strong industrial impact.

However, several areas require strengthening before the work is suitable for publication:

- Clarification and strengthening of the regulatory basis (KOSHA relevance)
While the manuscript clearly references KOSHA guidance (e.g., M-69-2012, C-C-23-2026, Article 256), the link between engineering outputs and regulatory obligations should be made more explicit. The authors should:

  - Provide clause-level citations and short extracts where possible

  - Clarify whether requirements are mandatory vs guidance

  - Demonstrate more explicitly how each obligation is not already covered in standard EPC workflows

- Reproducibility and methodological transparency
The architecture is well described conceptually, but the study would benefit from:

  - More detailed explanation of the calculation engines and orchestration logic

  - Clearer description of synthetic dataset generation

  - Formalisation of the RAG query and prompting strategy

- Validation and statistical rigor
The evaluation relies heavily on synthetic datasets with near-perfect scores. This is acceptable for a proof-of-feasibility study, but the authors should:

  - Clearly position results as implementation verification rather than predictive validation

  - Add uncertainty discussion or sensitivity analysis

  - Expand or better justify the retrieval benchmark size and representativeness

- Application scope – HAZOP clarification
The manuscript would benefit from clarifying that the proposed approach is best suited for:

  - Knowledge-based or AI-assisted HAZOP support, where structured data and rules can be leveraged

  - and not a replacement for conventional dynamic HAZOP, which remains a brainstorming-based, multidisciplinary workshop process relying on expert judgement and interaction

- Fitness-for-Service (FFS) vs EPC standards
The relationship between fitness-for-service (e.g., API 579) and EPC design standards (e.g., ASME, API design codes) should be explained more clearly. In particular:

  - How FFS outputs interact with regulatory obligations

  - Whether the system treats FFS as design validation, operational assessment, or both

  - How this affects compliance detection in the proposed framework

- Figures and structure improvements
The manuscript would benefit significantly from:

  - A system architecture diagram

  - A RAG workflow diagram

  - Improved visualisation of cross-discipline coupling logic

- Regulatory RAG compared with HAZOP, RBI and digital twin

| Method | Main purpose | Weakness | Regulatory RAG value |
| --- | --- | --- | --- |
| HAZOP | Identify process deviations | Often manual and workshop-based | Adds regulation-linked prompts and missing legal obligations |
| RBI | Optimize inspection based on risk | Focuses mainly on equipment degradation | Adds jurisdiction-specific inspection duties |
| Digital Twin | Live/near-live plant model | May not know legal obligations | Adds compliance intelligence layer |
| Regulatory RAG | Retrieve applicable laws/standards | Depends on corpus quality | Bridges engineering + legal compliance |

Best use is not to replace HAZOP/RBI, but to act as a compliance co-pilot:

- Before HAZOP: identify mandatory regulatory nodes

- During design review: detect missing legal requirements

- During RBI: check whether local inspection triggers are satisfied

- During digital twin operation: flag live compliance gaps

- Application in process safety terms

Regulatory RAG acts like a:

- Digital Compliance Auditor: is an automated system that checks whether your design, operation, or documentation complies with regulations—just like a human auditor would.

- Automated PSM checker: A system that verifies compliance with Process Safety Management (PSM) requirements automatically.

(PSM = SEVESO / OSHA / KOSHA frameworks)

- Real-time regulatory advisor: A system that gives instant regulatory guidance while you are designing or operating.

| Role | When |
| --- | --- |
| Auditor | After work is done |
| Advisor | While work is being done |

- Addition of acronyms and definitions section
Given the multidisciplinary nature of the paper, a dedicated section listing:

  - Acronyms (e.g., RAG, PSM, RBI, FFS, KOSHA, EPC)

  - Key definitions (e.g., jurisdiction compliance gap, K-voting, coupling validation)
would greatly improve readability and accessibility.